Introduction

In this lab, we will be using the Bassy3 board to leard about FPGA implementation. By using the Vivoda EDA tool, we will be able to flash a program onto the board to verify that it works correctly. The program is a voting machine with weighted inputs. One of the voters will receive a weight of 2, the second will receive a weight of 3 and the last will receive a weight of 4.

Design Methodology

For the design of this lab, I referred to the given material in the Lab Tutorial document from the CMPE 125 class. There are a total of 5 modules that were used in this lab, as described below in the Module Table. The use of the modules is seen in the top level design as shown in figure 1.

In addition to the 5 modules, there was a testbench file to compare the simulation waveform to.

The last item is the constraint file, voting_machine_fpga.xdc, which was used to set certain pins on the Bassy3 FPGA board to be linked to certain items in the modules.

Table 1: Module Table

| Module | Function |
| --- | --- |
| voting_machine_fpga.v | Top level module which combines all of the lower level modules to create a voting machine |
| voting_rule.v | Converts inputs to BCD (binary coded decimal) |
| clk_gen.v | Converts clock to 5KHz |
| bcd_to_7seg.v | Converts BCD number to match appropriate 7seg display value |
| led_mux.v | Multiplexer which choose a 7seg display |

The functions of voting_rule.v and bcd_to_7seg.v can be seen in table 2 and table 3 below.

Table 2. Function table for Module voting_rule.v

| Inputs(with weights) | Outputs (in BCD code) | Decimal Display |
| --- | --- | --- |
| (w1) (w2) (w3) | a b c d | 7-seg LED |
| 0 0 0 | 0 0 0 0 | 0 |
| 0 0 1 | 0 1 0 0 | 4 |
| 0 1 0 | 0 0 1 1 | 3 |
| 0 1 1 | 0 1 1 1 | 7 |
| 1 0 0 | 0 0 1 0 | 2 |
| 1 0 1 | 0 1 1 0 | 6 |
| 1 1 0 | 0 1 0 1 | 5 |
| 1 1 1 | 1 0 0 1 | 9 |

Table 3. Function Table for Module bcd_to_7seg.v

| Inputs (in BCD) | Outputs (active low) | Decimal Display |
| --- | --- | --- |
| BCD[3:0] | S6 S5 S4 S3 S2 S1 S0 | 7-seg LED |
| 0 0 0 0 | 1 0 0 0 0 0 0 | 0 |
| 0 1 0 0 | 0 0 0 1 1 0 1 | 4 |
| 0 0 1 1 | 0 1 1 0 0 0 0 | 3 |
| 0 1 1 1 | 1 1 1 1 0 0 0 | 7 |
| 0 0 1 0 | 0 1 0 0 1 0 0 | 2 |
| 0 1 1 0 | 1 0 0 0 0 1 0 | 6 |
| 0 1 0 1 | 0 0 1 0 0 1 0 | 5 |
| 0 0 0 0 | 0 0 1 0 0 0 0 | 9 |

Figure 1. Top level module of voting_machine_fpga.v

Simulation Results

For this lab, we had three weighted inputs, labeled w, n and o in the figure 2 simulation. These three weighted bits were converted to BCD by the voting_rule module and then converted to outputs for the 7 segment displays by the bcd_to_7seg module. If we look at the case of wno = 100, we see that abcd = 0010 which results in the number 2 showing up on the 7 segment display. By looking at the function tables in table 1 and table 2, we see that the simulation results are correct.

Also, a file called tb_voting_rule was created to verify that the simulation was correct, and both waveforms were the same.

Figure 2. Simulation waveform

FPGA Validation

To test the FPGA board, I used a variety of inputs to test the voting machine. Some of the inputs tested are shown in figures 3, 4 and 5.

First off, I tested an input of 1 0 0. Referring to table 2, we see that the output should be 2. This is seen in figure 3. I proceeded to turn the other switches on to test for cases of 1 1 0 and 1 1 1 respectively, and all testing scenarios proved to be correct.

Figure 3. Input of 1 0 0 on the FPGA board (2 + 0 + 0) results in 2

Figure 4. Input of 1 1 0 on the FPGA board (2 + 3 + 0) results in 5

Figure 5. Input of 1 1 1 on the FPGA board (2 + 3 + 4) results in 9

Conclusion

In this lab, I learned how to create a project in the Vivado Design Suite EDA and successfully turn the project into a bitstream which can then be loaded onto my Bassy3 FPGA board. One of the important takeaways is that I must validate my waveform with a testbench waveform to verify that all of the modules are working correctly. Once this has been verified to be true, the next step is to flash the board with the bitstream. After successfully flashing the board, I was able to verify that the board works by comparing it to the tables in tables 2 and 3. I was able to successfully complete the lab during the allotted time with a complete mark.

Appendix

Table 4: Source Code

voting_machine_fpga.v

module voting_machine_fpga ( input wire clk100MHz, input wire rst, input wire A, input wire B, input wire C, output wire A_led, output wire B_led, output wire C_led, output wire [3:0] LEDSEL, output wire [7:0] LEDOUT ); assign A_led = A; assign B_led = B; assign C_led = C; supply1 [7:0] vcc;. wire DONT_USE; wire clk_5KHz; wire [3:0] i; wire [7:0] vote; voting_rule VTR ( .w (A), .n (B), .o (C), .a (i[3]), .b (i[2]), .c (i[1]), .d (i[0]) ); clk_gen CLK ( .clk100MHz (clk100MHz),. .rst (rst), .clk_4sec (DONT_USE), .clk_5KHz (clk_5KHz) ); bcd_to_7seg BCD ( .BCD (i), .s (vote) ); led_mux LED ( .clk (clk_5KHz), .rst (rst), .LED3 (vcc), .LED2 (vcc),. .LED1 (vcc), .LED0 (vote), .LEDSEL (LEDSEL), .LEDOUT (LEDOUT) ); endmodule.

voting_rule.v

module voting_rule ( input wire w, input wire n, input wire o, output wire a, output wire b, output wire c, output wire d ); assign a = w&n&o; assign b = ~w&~n&o ¦ ~w&n&o ¦ w&~n&o ¦ w&n&~o; assign c = ~w&n&~o ¦ ~w&n&o ¦ w&~n&o ¦ w&~n&~o; assign d = ~w&n&~o ¦ ~w&n&o ¦ w&n&~o ¦ w&n&o; endmodule. .LED1 (vcc), .LED0 (vote), .LEDSEL (LEDSEL), .LEDOUT (LEDOUT) ); endmodule.

clk_gen.v

module clk_gen ( input wire clk100MHz, input wire rst, output reg clk_4sec, output reg clk_5KHz ); integer count1, count2; always @ (posedge clk100MHz) begin if (rst) begin count1 = 0; clk_4sec = 0; count2 = 0; clk_5KHz = 0; end else begin if (count1 == 200000000) begin clk_4sec = ~clk_4sec; count1 = 0; end if (count2 == 10000) begin clk_5KHz = ~clk_5KHz; count2 = 0; end count1 = count1 + 1; count2 = count2 + 1;. End End endmodule

bcd_to_7seg.v

module bcd_to_7seg ( input wire [3:0] BCD, output reg [7:0] s ); always @ (BCD) begin case (BCD) 4'd0: s = 8'b11000000; 4'd1: s = 8'b11111001; 4'd2: s = 8'b10100100; 4'd3: s = 8'b10110000; 4'd4: s = 8'b10011001; 4'd5: s = 8'b10010010; 4'd6: s = 8'b10000010; 4'd7: s = 8'b11111000; 4'd8: s = 8'b10000000; 4'd9: s = 8'b10010000; default: s = 8'b01111111; endcase end endmodule.

led_mux.v

module led_mux ( input wire clk, input wire rst, input wire [7:0] LED3, input wire [7:0] LED2, input wire [7:0] LED1, input wire [7:0] LED0, output wire [3:0] LEDSEL, output wire [7:0] LEDOUT ); reg [1:0] index; reg [11:0] led_ctrl; assign {LEDSEL, LEDOUT} = led_ctrl; always @ (posedge clk) index <= (rst) ? 2'b0 : (index + 2'd1); always @ (index, LED0, LED1, LED2, LED3) begin case (index) 4'd0: led_ctrl <= {4'b1110, LED0}; 4'd1: led_ctrl <= {4'b1101, LED1}; 4'd2: led_ctrl <= {4'b1011, LED2}; 4'd3: led_ctrl <= {4'b0111, LED3}; default: led_ctrl <= {8'b1111, 8'hFF};. endcase end endmodule.

tb_voting_rule.v

module tb_voting_rule; reg w; reg n; reg o; wire a; wire b; wire c; wire d; voting_rule DUT ( .w (w), .n (n), .o (o), .a (a), .b (b), .c (c), .d (d) ); initial begin o = 0; forever #5 o = ~o; end initial begin n = 0; forever #10 n = ~n; end initial begin w = 0; forever #20 w = ~w; end initial #40 $stop; initial $monitor( $time, " A=%b, B=%b, C=%b : a=%b, b=%b, c=%b, d=%b", w, n, o, a, b, c, d ); endmodule.

voting_machine_fpga.xdc

# Clock signal set_property -dict {PACKAGE_PIN W5 IOSTANDARD LVCMOS33} [get_ports {clk100MHz}]; create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports {clk100MHz}]; # Buttons set_property -dict {PACKAGE_PIN U18 IOSTANDARD LVCMOS33} [get_ports {rst}]; # Center Button # Switches set_property -dict {PACKAGE_PIN V17 IOSTANDARD LVCMOS33} [get_ports {C}]; # Switch 0 set_property -dict {PACKAGE_PIN V16 IOSTANDARD LVCMOS33} [get_ports {B}]; # Switch 1 set_property -dict {PACKAGE_PIN W16 IOSTANDARD LVCMOS33} [get_ports {A}]; # Switch 2 # LEDs set_property -dict {PACKAGE_PIN U16 IOSTANDARD LVCMOS33} [get_ports {C_led}]; # LED 0 set_property -dict {PACKAGE_PIN E19 IOSTANDARD LVCMOS33} [get_ports {B_led}]; # LED 1 set_property -dict {PACKAGE_PIN U19 IOSTANDARD LVCMOS33} [get_ports {A_led}]; # LED 2 # 7 Segment Display set_property -dict {PACKAGE_PIN W7 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[0]}]; # CA set_property -dict {PACKAGE_PIN W6 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[1]}]; # CB set_property -dict {PACKAGE_PIN U8 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[2]}]; # CC set_property -dict {PACKAGE_PIN V8 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[3]}]; # CD set_property -dict {PACKAGE_PIN U5 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[4]}]; # CE set_property -dict {PACKAGE_PIN V5 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[5]}]; # CF set_property -dict {PACKAGE_PIN U7 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[6]}]; # CG set_property -dict {PACKAGE_PIN V7 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[7]}]; # DP set_property -dict {PACKAGE_PIN U2 IOSTANDARD LVCMOS33} [get_ports {LEDSEL[0]}]; # AN0 set_property -dict {PACKAGE_PIN U4 IOSTANDARD LVCMOS33} [get_ports {LEDSEL[1]}]; # AN1 set_property -dict {PACKAGE_PIN V4 IOSTANDARD LVCMOS33} [get_ports {LEDSEL[2]}]; # AN2.