Introduction

The purpose of this lab is to familiarize ourselves with the hierarchical design methodology and design/verification /validation flow and the EDA tool. Our task was to create a 4-bit adder and run a simulation test via “inferred” design approach which required us to use the “+” sign. Then we designed and ran a 4-bit adder but this time using the carry look ahead method and implementing it on our BASYS3 board. Unlike the ripple carry adder, the carry look ahead adder is used because it removes the propagation delay that would be present in a ripple carry adder. It does this by computing all of the carries at the same time.

Design Methodology

When we designed the inferred method, we simply made the adder by using the ‘+’ sign and using an if statement to compare if it was greater than 15. If it was, we added a carry over bit and displayed the rest. However, when we made an adder using the Carry Look Ahead method, we had to compare each bit using an AND function to see if it had any carry over and using a XOR function to calculate the sum. Then we gathered that information and made a Carry Look Ahead module to calculate the 4 bit addition.

To calculate the carry, we used some relations from our adders to create our carryBits.v module, as shown in figure 7.

pi=ai⊕bi   (1)

gi=ai∙bi   (2)

ci+1=gi⊕pici   (3)

Utilizing formulas 1, 2 and 3, the carry for all of the bits in the CLA adders can be calculated, as shown below.

c1=g0+p0c0

c2=g1+p1g0+p1p0c0

c3=g2+p2g1+p2p1g0+p2p1p0c0

c4=g3+p3g2+p3p2g1+p3p2p1g0+p3p2p1p0c0

As seen in the equations above, none of the carry values depend on a previous carry value. All of the carries can be calculated in parallel vs in series as in the ripple adder which reduces the propagation delay. Unfortunately, there are some issues that arise with this method. If we were to calculate a 64 bit number, for example, there would be an enormous number of gates to computer all of the carries at the same time, and this would increase the manufacturing costs.

The modules are listed in table 1 below, and it describes the different modules used on this assignment.

Table 1. Module descriptions

| Module | Function |
| --- | --- |
| cla_adder.v | Performs a 4 bit addition w/ carry look ahead |
| carryBits.v | Calculates the carry bits in parallel |
| add_half.v | Calculates sum and carry over |
| bcd_to_7seg.v | Displays binary result on 7 seg LED |
| LEDSEL.v | Chooses which 7 seg to display to |
| inferred_adder.v | Performs a 4 bit addition operation in an “inferred” way |
| inferred_adder_self_checkingtb.v | Check all possible inputs of the inferred adder approach |
| clk_gen.v | Generates a clock signal to synchronize with |

As seen in figure 1, the top level module of the CLA utilizes 4 half bit adders (figure 2), and it computers the carry out in parallel using the carrybits module shown in figure 7.. This is very different from the top level design of the inferred adder module as shown in figure 6. Figures 3 through 5 show the modules used for our 7 segment display.

Figure 1: Carry look ahead structure diagram

Figure 2: add_half structure diagram

Figure 3: led_mux structure diagram

Figure 4:bcd_to_7seg structure diagram

Figure 5:clk_gen structure diagram

Figure 6:inferred adder structure diagram

Figure 7:carry bits structure diagram

Simulation results

Figure 7: Inferred method ALU waveform

Figure 8: Carry Look Ahead method ALU waveform

FPGA Validation

When making the constraint file for the BASYS3, we made the left two 7-segment LED show the sum of the two inputs, with a max value of 15. When the sum goes past 16, then the 3rd 7-segment LED will turn on showing the carry over bit since 4 bit can only represent up till 15. The left two LED will show the remainder of the sum. When the carry in button is pressed, we added 1 to the two inputs and the most right LED will turn on, indicating that it is on.

Figure 9: Left-No switches on, Right-B[2]B[0] and A[3]A[0] on

Figure 10: Left-B[3]B[2]B[0] and A[3]A[0] on,, Right-B[3]B[2]B[0] and A[3]A[0]and carry switch on

Conclusion

The purpose of this lab is to familiarize ourselves with the hierarchical design methodology and design/verification /validation flow and the EDA tool, as well as learning about the effect of propagation delay in an adder. We were successful in designing a 4-bit adder and running a simulation test via “inferred” design approach which required us to use the “+” sign. We were also successful designing and running a 4-bit adder using the carry look ahead method and implementing it on our BASYS3 board.

In designing our adder, we learned about the differences between a carry look ahead approach and a ripple carry approach. When we do the CLA method, we can compute all of the carry bits in parallel. Although this is important in reducing propagation delay, its disadvantage is that it requires a lot of gates if we were to make a 64 bit adder. To get the best of both propagation delay and the number of gates, it is possible to make multiple 4 bit CLA adders and then link these together using the ripple carry method to get to 64 bits. This was not a part of the lab, but our design gave us insight towards what we could do in a higher level design. Overall, the lab was successful and we learned how to program a button in our constraint file, which we had not previously done before.

Appendix

inferred_adder.v

module inferred_adder(sum, c_out, a, b); output reg [3:0] sum; output reg c_out; input wire [3:0] a; input wire [3:0] b; always @ (a, b) begin if(a + b > 15) begin c_out = 1; sum = a + b; end else begin c_out = 0; sum = a + b; end end endmodule

inferred_adder_self_checkingtb.v

`timescale 1ns / 1ps module self_checking_tb; wire [3:0] sum_tb; wire c_out_tb; reg exp_c_out; reg [3:0] sum_exp; reg [3:0] a_tb; reg [3:0] b_tb; inferred_adder DUT( .sum(sum_tb), .c_out(c_out_tb), .a(a_tb), .b(b_tb) ); integer i, j; integer SumErrorCount; integer CarryErrorCount; initial begin SumErrorCount = 0; CarryErrorCount = 0; for(i = 4'b0000; i <= 4'b1111; i = i + 4'b0001) begin a_tb = i; for(j = 4'b0000; j <= 4'b1111; j = j + 4'b0001) begin b_tb = j; if(sum_exp[3:0] === sum_tb[3:0]) begin end else begin SumErrorCount = SumErrorCount + 1; $display ("sum_exp ="); $display (sum_exp); $display ("sum_tb = "); $display (sum_tb); end if(exp_c_out === c_out_tb) begin end else begin CarryErrorCount = CarryErrorCount + 1; $display ("carry exp ="); $display (exp_c_out); $display ("c_out_tb = "); $display (c_out_tb); end sum_exp = i + j; exp_c_out = 0; if(i + j > 15)begin exp_c_out = 1; end #5; end end $display ("Simulation Finished"); $finish; end endmodule

clk_gen.v

`timescale 1ns / 1ps module tb_cla_adder; reg [3:0] tb_A; reg [3:0] tb_B; reg tb_Cin; wire tb_Cout; wire [3:0] tb_S; reg [3:0]tb_S_expected; integer error = 0; integer i, j; cla_adder DUT(.A(tb_A), .B(tb_B), .C_in(tb_Cin), .C_out(tb_Cout), .Sum(tb_S)); initial begin for(i = 0; i < 16; i = i + 1) begin for(j = 0; j < 16; j = j + 1) begin tb_Cin = 0; tb_A = i; tb_B = j; tb_S_expected = i + j; #1; if(tb_S != tb_S_expected) begin $display("Error"); error =+1 ; end else begin end end end end endmodule

clk_gen.v

module clk_gen ( input wire clk100MHz, input wire rst, output reg clk_4sec, output reg clk_5KHz ); integer count1, count2; always @ (posedge clk100MHz) begin if (rst) begin count1 = 0; clk_4sec = 0; count2 = 0; clk_5KHz = 0; end else begin if (count1 == 200000000) begin clk_4sec = ~clk_4sec; count1 = 0; end if (count2 == 10000) begin clk_5KHz = ~clk_5KHz; count2 = 0; end count1 = count1 + 1; count2 = count2 + 1; end end endmodule

bcd_to_7seg.v

module bcd_to_7seg ( input wire [3:0] BCD, output reg [7:0] s ); always @ (BCD) begin case (BCD) 4'd0: s = 8'b11000000; 4'd1: s = 8'b11111001; 4'd2: s = 8'b10100100; 4'd3: s = 8'b10110000; 4'd4: s = 8'b10011001; 4'd5: s = 8'b10010010; 4'd6: s = 8'b10000010; 4'd7: s = 8'b11111000; 4'd8: s = 8'b10000000; 4'd9: s = 8'b10010000; default: s = 8'b01111111; endcase end endmodule

LEDSEL.v

module LEDSEL ( input wire clk, input wire rst, input wire [7:0] LED3, input wire [7:0] LED2, input wire [7:0] LED1, input wire [7:0] LED0, output wire [3:0] LEDOUT, output wire [7:0] LEDOUT ); reg [1:0] index; reg [11:0] led_ctrl; assign {LEDOUT, LEDOUT} = led_ctrl; always @ (posedge clk) index <= (rst) ? 2'b0 : (index + 2'd1); always @ (index, LED0, LED1, LED2, LED3) begin case (index) 4'd0: led_ctrl <= {4'b1110, LED0}; 4'd1: led_ctrl <= {4'b1101, LED1}; 4'd2: led_ctrl <= {4'b1011, LED2}; 4'd3: led_ctrl <= {4'b0111, LED3}; default: led_ctrl <= {8'b1111, 8'hFF}; endcase end endmodule

cla_adder.v

module cla_adder( input [3:0]A, input [3:0]B, input C_in, output C_out, output [3:0] Sum ); wire [3:0] P; wire [3:0] G; wire [4:0] C; carryBits cBits(.P(P), .G(G), .C_in(C_in), .C_out(C)); Add_half AH0(.a(A[0]), .b(B[0]), .c_out(P[0]), .sum(G[0])); Add_half AH1(.an(A[1]), .b(B[1]), .c_out(P[1]), .sum(G[1])); Add_half AH2(.a(A[2]), .b(B[2]), .c_out(P[2]), .sum(G[2])); Add_half AH3(.a(A[3]), .b(B[3]), .c_out(P[3]), .sum(G[3])); assign Sum[0] = P[0] ^ C[0]; assign Sum[1] = P[1] ^ C[1]; assign Sum[2] = P[2] ^ C[2]; assign Sum[3] = P[3] ^ C[3]; assign C_out = C[4]; endmodule

cla_adder_fpga.v

`timescale 1ns / 1ps module cla_adder_fpga( input wire clk100mhz, input wire cin_button, input wire [3:0]inputA, input wire [3:0]inputB, output reg [3:0]outputA, output reg [3:0]outputB, output wire [7:0]LEDOUT, output wire [3:0]LEDSEL ); wire [3:0]sum_out; wire carryOut; wire dontuse; Wire clk_5KHz; Clk_gen CLK ( .clk100MHz (clk100MHz), .rst (rst), .clk_4sec (dontuse), .clk_5KHz (clk_5KHz) ); Bcd_to_7seg BCD ( .BCD (LEDSEL), .s (LEDOUT) ); Led_mux LED ( .clk (clk_5KHz), .rst (rst), .LED3 (LEDOUT), .LED2 (LEDOUT), .LED1 (LEDOUT), .LED0 (LEDOUT). .LEDSEL (LEDSEL), .LEDOUT (LEDOUT) ); carryBits cBits( .P(inputA), .G(inputB), .C_in(cin_button), .C_out(carryOut), ); always @ (inputA, inputB, cin_button) begin outputA = inputA; outputB = inputB; if(cin_button) begin led0 = 4'b0001; end else begin led0 = 4'b1111; end if(carryOut) begin led1 = 4'b0001; end else begin led1 = 4'b1111; end if(sum_out >= 10) begin led2 = 4'b0001; led3 = sum_out - 10; end else begin led2 = 4'b1111; led3 = sum_out; end end endmodule

carryBits.v

module carryBits( input [3:0]P, input [3:0]G, input C_in, output[4:0]C_out ); assign C_out[0] = C_in; assign C_out[1] = G[0] ¦ P[0] & C_in; assign C_out[2] = G[1] ¦ P[1] & G[0] ¦ P[1] & P[0] & C_in; assign C_out[3] = G[2] ¦ P[2] & G[1] ¦ P[2] & P[1] & G[0] ¦ P[2] & P[1] & P[0] & C_in; assign C_out[4] = G[3] ¦ P[3] & G[2] ¦ P[3] & P[2] & G[1] ¦ P[3] & P[2] & P[1] & G[0] ¦ P[3] & P[2] & P[1] & P[0] & C_in; endmodule

Add_half.v

`timescale 1ns / 1ps module Add_half( input a,b; output c_out, sum; ); assign sum = a ^ b; assign c_out = a & b; endmodule

cla_adder_fpga.xdc

# Clock set_property -dict {PACKAGE_PIN W5 IOSTANDARD LVCMOS33} [get_ports {clk100mhz}]; create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports {clk100mhz}]; # Button set_property -dict {PACKAGE_PIN T18 IOSTANDARD LVCMOS33} [get_ports {cin_button}]; #Button - cin # Switches set_property -dict {PACKAGE_PIN V17 IOSTANDARD LVCMOS33} [get_ports {inputB[0]}]; # Switch 0 set_property -dict {PACKAGE_PIN V16 IOSTANDARD LVCMOS33} [get_ports {inputB[1]}]; # Switch 1 set_property -dict {PACKAGE_PIN W16 IOSTANDARD LVCMOS33} [get_ports {inputB[2]}]; # Switch 2 set_property -dict {PACKAGE_PIN W17 IOSTANDARD LVCMOS33} [get_ports {inputB[3]}]; # Switch 3 set_property -dict {PACKAGE_PIN W2 IOSTANDARD LVCMOS33} [get_ports {inputA[0]}]; # Switch 4 set_property -dict {PACKAGE_PIN U1 IOSTANDARD LVCMOS33} [get_ports {inputA[1]}]; # Switch 5 set_property -dict {PACKAGE_PIN T1 IOSTANDARD LVCMOS33} [get_ports {inputA[2]}]; # Switch 6 set_property -dict {PACKAGE_PIN R2 IOSTANDARD LVCMOS33} [get_ports {inputA[3]}]; # Switch 7 # LEDs set_property -dict {PACKAGE_PIN U16 IOSTANDARD LVCMOS33} [get_ports {outputB[0]}]; # LED 0 set_property -dict {PACKAGE_PIN E19 IOSTANDARD LVCMOS33} [get_ports {outputB[1]}]; # LED 1 set_property -dict {PACKAGE_PIN U19 IOSTANDARD LVCMOS33} [get_ports {outputB[2]}]; # LED 2 set_property -dict {PACKAGE_PIN V19 IOSTANDARD LVCMOS33} [get_ports {outputB[3]}]; # LED 3 set_property -dict {PACKAGE_PIN P3 IOSTANDARD LVCMOS33} [get_ports {outputA[0]}]; # LED 4 set_property -dict {PACKAGE_PIN N3 IOSTANDARD LVCMOS33} [get_ports {outputA[1]}]; # LED 5 set_property -dict {PACKAGE_PIN P1 IOSTANDARD LVCMOS33} [get_ports {outputA[2]}]; # LED 6 set_property -dict {PACKAGE_PIN L1 IOSTANDARD LVCMOS33} [get_ports {outputA[3]}]; # LED 7 # 7 Segment Display set_property -dict {PACKAGE_PIN U2 IOSTANDARD LVCMOS33} [get_ports {LEDSEL[0]}]; # mux 0 set_property -dict {PACKAGE_PIN U4 IOSTANDARD LVCMOS33} [get_ports {LEDSEL[1]}]; # mux 1 set_property -dict {PACKAGE_PIN V4 IOSTANDARD LVCMOS33} [get_ports {LEDSEL[2]}]; # mux 2 set_property -dict {PACKAGE_PIN W4 IOSTANDARD LVCMOS33} [get_ports {LEDSEL[3]}]; # mux 3 set_property -dict {PACKAGE_PIN W7 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[0]}]; # out 0 set_property -dict {PACKAGE_PIN W6 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[1]}]; # out 1 set_property -dict {PACKAGE_PIN U8 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[2]}]; # out 2 set_property -dict {PACKAGE_PIN V8 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[3]}]; # out 3 set_property -dict {PACKAGE_PIN U5 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[4]}]; # out 4 set_property -dict {PACKAGE_PIN V5 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[5]}]; # out 5 set_property -dict {PACKAGE_PIN U7 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[6]}]; # out 6 set_property -dict {PACKAGE_PIN V7 IOSTANDARD LVCMOS33} [get_ports {LEDOUT[7]}]; # out 7