Introduction

In this lab, we are assigned to build a system level design with emphasis on the control portion. To fulfill our goal, we build a calculator with different states and a go signal to indicate when our CPU can start doing the calculation. Then we made a couple testbench and also a self checking testbench to check our progress.

Design Methodology

The design of this lab first begins with understanding the timing behind how our small calculator works. First, we need to load two numbers into the register. Since we can only load a single number at a time, we must use at least 2 clock cycles for this portion of the calculator. Once the numbers have been loaded into the register, we can then follow through with performing our operation.

In order to use the calculator, a state machine must be generated through the control unit to correctly perform the calculation each time a number is input. The control unit will output the appropriate control signals depending on which state it is in. To initialize the state machine, the go signal must be a logic high. This initializes the control signals for the calculator and then it sends the state machine through each of its different states. Initially, the first 3 states are the same regardless of the operation. After this, it will go into either state 4, 5, 6 or 7 depending on the OP code given, which decides which operation to perform. The final state is used to output the result to the 7 segment displays, and then it will cycle in state 1 repeatedly until a new go signal is given.

In table 1 shown below, the modules used in the design of our small calculator shown.

Table 1. Module Definitions

| Modules | Function |
| --- | --- |
| small_calculator_dp | Implements calculator with datapaths |
| small_calculator_cu | Implements calculator with control unit |
| control_unit.v | Implements control unit for calculator |
| mux4.v | Implements 4 to 1 mux |
| register_file.v | Implements register file |
| alu.v | Implements alu to perform calculation |
| mux2.v | Implements 2 to 1 mux |
| clk_gen.v | Generates clock signal |
| button_debouncer.v | Generates clock signal via button press |
| sevenSegDisplay.v | Generates decimal number on 7-segment LED on FPGA board |
| self_checking_tb.v | Testbench for calculator with datapath |
| go_clk_tb.v | Testbench for control unit functionality |
| fsm_calc_tb.v | Full self checking testbench for calculator with control unit |

Schematic 1. Schematic of the small calculator high level design

Simulation Results

Task 1:

The first waveform shows the calculation being done using input 1, input 2, and c which determines the operation. Here, we are determining the correctness of our datapath when we are doing the calculations.

Figure 1. Waveform from task 1

Task 2:

The waveform shown below is a simple testbench to show that our control unit is working as intended. By passing in clock signals without the go active, the state should not advance anywhere. However, after setting go to active and sending clock signals, then we are able to see the control unit advance to other states. The state it advances to is determined by what c is, which is responsible for choosing what operation the ALU is going to perform.

Figure 2. Waveform from Control Unit

Task 3:

The final waveform shows what we combined in the last 2 weeks. In it, we see the calculation done using the two inputs and different operations determined by c. With different c values, the state will range from 4 to 7. The done flag will be active when the state reaches 8. The error counter will increment when either the calculation is not equal to the expected value, or if the done flag sets or resets at the wrong state.

Figure 3. Waveform from Calculator

FPGA Validation

To validate the calculator on the FPGA board, figures 4 through 7 will show our results.

The two leftmost switches are our control switches, the 4th led from the left is the “done” signal, the center button is the debounced clock button and the right button is the “go” signal. The 8 rightmost switches represent the two 4 bit inputs to our calculator.

On the 7 segment display, the two leftmost numbers represent the output, and the rightmost number represents the state while the second to the right is not used and always displays a zero.

In figure 4 below, an addition operation is shown. Input 2 is 0011b and input 1 is 0101b which is 5 + 3. In the eighth state, the output is displayed and the done signal is active. The output shows the correct result.

Figure 4. Addition of the numbers 3 and 5

In figure 5, a subtraction operation is shown because the OP code is 01. Using the same numbers, the operation is 5 - 3 = 2. The correct output is shown again.

Figure 5. Subtraction of the numbers 3 and 5

In figure 6, an AND operation is shown because the OP code is 10. Using the same numbers, the operation is 0101b • 0011b = 0001b = 1. The correct output is shown again.

Figure 6. AND of the numbers 3 and 5

In figure 7, an XOR operation is shown because the OP code is 10. Using the same numbers, the operation is 0101b ^ 0011b = 0110b = 6. The correct output is shown again.

Figure 7. XOR of the numbers 3 and 5

From our FPGA validation, our calculations were shown to be correct.

Conclusion

This lab was successful because we were able to build a system level design with emphasis on the control portion. We also built a calculator with different states and a go signal to indicate when our CPU can start doing the calculation. At the end, we made testbenches to verify the calculator and the control unit and also a self checking testbench to check our progress.

Overall, we were able to complete all four tasks of the lab. We also implemented the calculator with the eight different stages on the FPGA board. To do this we added a debounced button to manually control our clock signal while using a state machine to generate the control signals. The control unit made a state machine to appropriately apply the correct timing to our calculator. This lab expanded on previous labs where we used modules such as registers and it required knowledge on how to appropriately add information into a register and also knowledge on how to extract this information in the correct sequence.

Appendix

small_calculator_dp.v

module small_calculator_dp ( input wire clk, input wire go, input wire [3:0] in1, input wire [3:0] in2, input wire [1:0] c, output wire [3:0] out, output wire done, output wire [3:0] state ); wire [3:0] mux1out; wire [3:0] douta; wire [3:0] doutb; wire [3:0] aluout; wire [1:0] s1; wire [1:0] wa; wire [1:0] raa; wire [1:0] rab; wire we; wire rea; wire reb; wire s2; wire [1:0] cout_cu; control_unit cu( .go (go), .clk(clk), .c(c), .s1(s1), .wa(wa), .raa(raa), .rab(rab), .c_out(cout_cu), .we(we), .rea(rea), .reb(reb), .s2(s2), .done(done), .state(state) ); mux4 #( .WIDTH(4) ) MUX1 ( .sel (s1), .in0 (4'b0), .in1 (in1), .in2 (in2), .in3 (aluout), .out (mux1out) ); register_file RF ( .clk (clk), .rea (rea), .reb (reb), .we (we), .raa (raa), .rab (rab), .wa (wa), .din (mux1out), .douta (douta), .doutb (doutb) ); alu ALU ( .c (cout_cu), .in0 (douta), .in1 (doutb), .alu_out (aluout) ); mux2 #( .WIDTH(4) ) MUX2 ( .sel (s2), .in0 (4'b0), .in1 (aluout), .out (out) ); endmodule

register_file.v

module register_file ( input wire clk, input wire rea, input wire reb, input wire we, input wire [1:0] raa, input wire [1:0] rab, input wire [1:0] wa, input wire [3:0] din, output wire [3:0] douta, output wire [3:0] doutb ); reg [3:0] RegFile[3:0]; always @ (posedge clk) begin if(we) begin RegFile[wa] <= din; end else begin RegFile[wa] <= RegFile[wa]; end end assign douta = (rea == 1'b1) ? RegFile[raa] : 4'b0; assign doutb = (reb == 1'b1) ? RegFile[rab] : 4'b0; endmodule

mux4.v

module mux4 #(parameter WIDTH = 4) ( input wire [1:0] sel, input wire [WIDTH-1:0] in0, input wire [WIDTH-1:0] in1, input wire [WIDTH-1:0] in2, input wire [WIDTH-1:0] in3, output reg [WIDTH-1:0] out ); always @ (*) begin case(sel) 2'b00: out = in0; 2'b01: out = in1; 2'b10: out = in2; 2'b11: out = in3; endcase end endmodule

mux2.v

module mux2 #(WIDTH = 4) ( input wire sel, input wire [WIDTH-1:0] in0, input wire [WIDTH-1:0] in1, output wire [WIDTH-1:0] out ); assign out = (sel == 1'b1) ? in1 : in0; endmodule

alu.v

module alu ( input wire [1:0] c, input wire [3:0] in0, input wire [3:0] in1, output reg [3:0] alu_out ); always @ (in0, in1, c) begin case(c) 2'b00: alu_out = in0 + in1; 2'b01: alu_out = in0 - in1; 2'b10: alu_out = in0 & in1; 2'b11: alu_out = in0 ^ in1; endcase end endmodule

button_debouncer.v

module button_debouncer #(parameter depth = 16) ( input wire clk, /* 5 KHz clock */ input wire button, /* Input button from constraints */ output reg debounced_button ); localparam history_max = (2**depth)-1; /* History of sampled input button */ reg [depth-1:0] history; always @ (posedge clk) begin /* Move history back one sample and insert new sample */ history <= { button, history[depth-1:1] }; /* Assert debounced button if it has been in a consistent state throughout history */ debounced_button <= (history == history_max) ? 1'b1 : 1'b0; end endmodule

clk_gen.v

`timescale 1ns / 1ps module clk_gen ( input wire clk100MHz, input wire rst, output reg clk_4sec, output reg clk_5KHz ); integer count1, count2; always @ (posedge clk100MHz) begin if (rst) begin count1 = 0; clk_4sec = 0; count2 = 0; clk_5KHz = 0; end else begin if (count1 == 200000000) begin clk_4sec = ~clk_4sec; count1 = 0; end if (count2 == 10000) begin clk_5KHz = ~clk_5KHz; count2 = 0; end count1 = count1 + 1; count2 = count2 + 1; end end endmodule

sevenSegDisplay.v

module sevenSegDisplay( input clk100MHz, input [3:0]led0, input [3:0]led1, input [3:0]led2, input [3:0]led3, output reg [7:0]led_out, output reg [3:0]led_pins ); reg clk_5KHz; reg [1:0]led_sel; reg [3:0]led_out_bcd; integer count1, count2; always @ (posedge clk100MHz) begin if(count1 == 200000000) begin count1 = 0; end if (count2 == 10000) begin clk_5KHz = ~clk_5KHz; count2 = 0; end count1 = count1 + 1; count2 = count2 + 1; end always @ (posedge clk_5KHz) begin led_sel = led_sel + 1; case(led_sel) 2'b00: begin led_pins = 4'b0111; led_out_bcd = led0; end 2'b01: begin led_pins = 4'b1011; led_out_bcd = led1; end 2'b10: begin led_pins = 4'b1101; led_out_bcd = led2; end 2'b11: begin led_pins = 4'b1110; led_out_bcd = led3; end endcase case(led_out_bcd) 4'd0: led_out = 8'b11000000; 4'd1: led_out = 8'b11111001; 4'd2: led_out = 8'b10100100; 4'd3: led_out = 8'b10110000; 4'd4: led_out = 8'b10011001; 4'd5: led_out = 8'b10010010; 4'd6: led_out = 8'b10000010; 4'd7: led_out = 8'b11111000; 4'd8: led_out = 8'b10000000; 4'd9: led_out = 8'b10010000; 4'b10: led_out = 8'b01111111; default: led_out = 8'b11111111; endcase end endmodule

selfchecking_tb.v

`timescale 1ns / 1ps module self_checking_tb; reg clk_tb; // reg [3:0] in1_tb; // reg [3:0] in2_tb; // reg [1:0] s1_tb; // reg [1:0] wa_tb; // reg [1:0] raa_tb;// reg [1:0] rab_tb;// reg [1:0] c_tb; //alu control reg we_tb; //write enable reg rea_tb; //read enable a reg reb_tb; //read enable b reg s2_tb; wire [3:0] out_tb; small_calculator_dp DUT( .clk(clk_tb), .in1(in1_tb), .in2(in2_tb), .s1(s1_tb), .wa(wa_tb), .raa(raa_tb), .rab(rab_tb), .c(c_tb), .we(we_tb), .rea(rea_tb), .reb(reb_tb), .s2(s2_tb), .out(out_tb) ); task clock; begin clk_tb = 0; #2; clk_tb = 1; #2; end endtask initial begin clk_tb = 0; rea_tb = 0; reb_tb = 0; raa_tb = 2'b00; rab_tb = 2'b00; c_tb = 2'b00; s2_tb = 0; in1_tb = 4'b0111; //7 in2_tb = 4'b0001; //1 clk_tb = 0; /////////////////////////////////////////////////////////////////////////////// wa_tb = 2'b00; s1_tb = 2'b00; we_tb = 0; clock; we_tb = 1; wa_tb = 2'b01; //2nd clock cycle s1_tb = 2'b01; clock; s1_tb = 2'b10; wa_tb = 2'b10; clock; s1_tb = 2'b00; we_tb = 0; wa_tb = 2'b00; clock; //add we_tb = 1; rea_tb = 1; reb_tb = 1; raa_tb = 2'b01; rab_tb = 2'b10; c_tb = 2'b00; s1_tb = 2'b11; wa_tb = 2'b11; clock; s1_tb = 2'b00; wa_tb = 2'b00; we_tb = 0; raa_tb = 2'b11; rab_tb = 2'b11; c_tb = 2'b10; s2_tb = 1; clock; //subtract we_tb = 1; rea_tb = 1; reb_tb = 1; raa_tb = 2'b01; rab_tb = 2'b10; c_tb = 2'b01; s1_tb = 2'b11; wa_tb = 2'b11; clock; s1_tb = 2'b00; wa_tb = 2'b00; we_tb = 0; raa_tb = 2'b11; rab_tb = 2'b11; c_tb = 2'b10; s2_tb = 1; clock; //and we_tb = 1; rea_tb = 1; reb_tb = 1; raa_tb = 2'b01; rab_tb = 2'b10; c_tb = 2'b10; s1_tb = 2'b11; wa_tb = 2'b11; clock; s1_tb = 2'b00; wa_tb = 2'b00; we_tb = 0; raa_tb = 2'b11; rab_tb = 2'b11; c_tb = 2'b10; s2_tb = 1; clock; //xor we_tb = 1; rea_tb = 1; reb_tb = 1; raa_tb = 2'b01; rab_tb = 2'b10; c_tb = 2'b11; s1_tb = 2'b11; wa_tb = 2'b11; clock; s1_tb = 2'b00; wa_tb = 2'b00; we_tb = 0; raa_tb = 2'b11; rab_tb = 2'b11; c_tb = 2'b10; s2_tb = 1; clock; end endmodule

go_clk_tb.v

`timescale 1ns / 1ps module go_clk_tb; reg clk_tb; reg go_tb; reg [3:0] in1_tb; reg [3:0] in2_tb; reg [1:0] c_tb=0; wire [3:0] out_tb; wire done_tb; wire [3:0] state_tb; small_calculator_dp DUT( .clk(clk_tb), .go(go_tb), .in1(in1_tb), .in2(in2_tb), .c(c_tb), .out(out_tb), .done(done_tb), .state(state_tb) ); task clock; begin clk_tb = 0; #2; clk_tb = 1; #2; end endtask integer i; initial begin for(i=0; i<5; i=i+1) begin go_tb=0; c_tb=c_tb+1; clock; clock; clock; clock; go_tb=1; clock; clock; clock; clock; clock; clock; end end endmodule

fsm_calc_tb.v

`timescale 1ns / 1ps module fsm_calc_tb; reg clk_tb; reg go_tb; reg [3:0] in1_tb; reg [3:0] in2_tb; reg [1:0] c_tb; wire [3:0] out_tb; wire done_tb; wire [3:0] state_tb; integer error=0; small_calculator_dp DUT( .clk(clk_tb), .go(go_tb), .in1(in1_tb), .in2(in2_tb), .c(c_tb), .out(out_tb), .done(done_tb), .state(state_tb) ); task clock; begin clk_tb = 0; #2; clk_tb = 1; #2; end endtask //go = 0 // couple clock //go =1 //couple task checkDone; begin if(4 <= state_tb ¦ state_tb <= 8) begin if(done_tb==0) begin error=error+1; end end if(0 <= state_tb ¦ state_tb <= 3) begin if(done_tb==1) begin error=error+1; end end end endtask integer i; integer j; integer k; reg[3:0] expected; initial begin for(i=3; i<16; i=i+1) begin for(j=6; j<16; j=j+1) begin for(k=0; k<4; k=k+1) begin in1_tb=i; in2_tb=j; c_tb=k; go_tb=1; clock; clock; clock; clock; clock; clock; //check add if(k==0) begin expected=i+j; if(expected != out_tb) begin error=error+1; end end //check sub if(k==1) begin expected=i-j; if(expected != out_tb) begin error=error+1; end end //check and if(k==2) begin expected=i&j; if(expected != out_tb) begin error=error+1; end end //check xor if(k==3) begin expected=i^j; if(expected != out_tb) begin error=error+1; end end //checkDone; go_tb=0; end end end end endmodule

system-constraint.xdc

# Clock input set_property -dict {PACKAGE_PIN W5 IOSTANDARD LVCMOS33} [get_ports {clk_FPGA}]; create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports {clk_FPGA}]; # Button (debouncer and go) set_property -dict {PACKAGE_PIN U18 IOSTANDARD LVCMOS33} [get_ports {button_clk}]; # Center Button set_property -dict {PACKAGE_PIN T17 IOSTANDARD LVCMOS33} [get_ports {button_go}]; #Right button # Input Switches set_property -dict {PACKAGE_PIN V17 IOSTANDARD LVCMOS33} [get_ports {Din1[0]}]; # Din10 set_property -dict {PACKAGE_PIN V16 IOSTANDARD LVCMOS33} [get_ports {Din1[1]}]; # Din11 set_property -dict {PACKAGE_PIN W16 IOSTANDARD LVCMOS33} [get_ports {Din1[2]}]; # Din12 set_property -dict {PACKAGE_PIN W17 IOSTANDARD LVCMOS33} [get_ports {Din1[3]}]; # Din13 set_property -dict {PACKAGE_PIN W15 IOSTANDARD LVCMOS33} [get_ports {Din2[0]}]; # Din20 set_property -dict {PACKAGE_PIN V15 IOSTANDARD LVCMOS33} [get_ports {Din2[1]}]; # Din21 set_property -dict {PACKAGE_PIN W14 IOSTANDARD LVCMOS33} [get_ports {Din2[2]}]; # Din22 set_property -dict {PACKAGE_PIN W13 IOSTANDARD LVCMOS33} [get_ports {Din2[3]}]; # Din23 set_property -dict {PACKAGE_PIN R2 IOSTANDARD LVCMOS33} [get_ports {c_fpga[1]}]; # c_fpga0 set_property -dict {PACKAGE_PIN T1 IOSTANDARD LVCMOS33} [get_ports {c_fpga[0]}]; # c_fpga1 # LED Output set_property -dict {PACKAGE_PIN U16 IOSTANDARD LVCMOS33} [get_ports {Dout1[0]}]; # Dout10 set_property -dict {PACKAGE_PIN E19 IOSTANDARD LVCMOS33} [get_ports {Dout1[1]}]; # Dout11 set_property -dict {PACKAGE_PIN U19 IOSTANDARD LVCMOS33} [get_ports {Dout1[2]}]; # Dout12 set_property -dict {PACKAGE_PIN V19 IOSTANDARD LVCMOS33} [get_ports {Dout1[3]}]; # Dout13 set_property -dict {PACKAGE_PIN W18 IOSTANDARD LVCMOS33} [get_ports {Dout2[0]}]; # Dout20 set_property -dict {PACKAGE_PIN U15 IOSTANDARD LVCMOS33} [get_ports {Dout2[1]}]; # Dout21 set_property -dict {PACKAGE_PIN U14 IOSTANDARD LVCMOS33} [get_ports {Dout2[2]}]; # Dout22 set_property -dict {PACKAGE_PIN V14 IOSTANDARD LVCMOS33} [get_ports {Dout2[3]}]; # Dout23 set_property -dict {PACKAGE_PIN P3 IOSTANDARD LVCMOS33} [get_ports {done_LED}]; # done LED set_property -dict {PACKAGE_PIN P1 IOSTANDARD LVCMOS33} [get_ports {c_LED[0]}]; # c LED set_property -dict {PACKAGE_PIN L1 IOSTANDARD LVCMOS33} [get_ports {c_LED[1]}]; # c LED # 7 Segment Display set_property -dict {PACKAGE_PIN W7 IOSTANDARD LVCMOS33} [get_ports {led_out_fpga[0]}]; # CA set_property -dict {PACKAGE_PIN W6 IOSTANDARD LVCMOS33} [get_ports {led_out_fpga[1]}]; # CB set_property -dict {PACKAGE_PIN U8 IOSTANDARD LVCMOS33} [get_ports {led_out_fpga[2]}]; # CC set_property -dict {PACKAGE_PIN V8 IOSTANDARD LVCMOS33} [get_ports {led_out_fpga[3]}]; # CD set_property -dict {PACKAGE_PIN U5 IOSTANDARD LVCMOS33} [get_ports {led_out_fpga[4]}]; # CE set_property -dict {PACKAGE_PIN V5 IOSTANDARD LVCMOS33} [get_ports {led_out_fpga[5]}]; # CF set_property -dict {PACKAGE_PIN U7 IOSTANDARD LVCMOS33} [get_ports {led_out_fpga[6]}]; # CG set_property -dict {PACKAGE_PIN V7 IOSTANDARD LVCMOS33} [get_ports {led_out_fpga[7]}]; # DP set_property -dict {PACKAGE_PIN U2 IOSTANDARD LVCMOS33} [get_ports {led_pins_fpga[0]}]; # AN0 set_property -dict {PACKAGE_PIN U4 IOSTANDARD LVCMOS33} [get_ports {led_pins_fpga[1]}]; # AN1 set_property -dict {PACKAGE_PIN V4 IOSTANDARD LVCMOS33} [get_ports {led_pins_fpga[2]}]; # AN2 set_property -dict {PACKAGE_PIN W4 IOSTANDARD LVCMOS33} [get_ports {led_pins_fpga[3]}]; # AN3